## Source: Hollman et Al. (2015) https://www.sciencedirect.com/science/article/pii/S1352231015300145#ec1

## Supplementary Information

# Table S1: Summary of the Air Quality Benefits of LEZs Identified From Monitoring Data

| City | Reduction in Long Term Concentrations Due to LEZ (%) |  |  |  |  |  |  |  | Notes | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PM10 | PM2.5 | PM1 | BC | EC | Abs. | NOx | NO2 |  |  |
| Berlin, Manheim, Stuttgart, Tubingen, Ludwigsburg | No effect |  |  |  |  |  |  |  | Comparison of cities with and without LEZs | Nierderemaier, 2009, cited in Cyrys et al., 2014 |
| 17 German cities with LEZs |  |  |  |  |  |  |  | Up to 4% | Matched quadruplets for before and after LEZ and within LEZ and at reference stations. LEZ Stage1. | Morfeld et al., 2014 |
| Berlin, Cologne | 5-7% |  |  |  |  |  |  |  | Comparison of annual average concentrations | Bruckmann and Lutz, 2010, cited in Cyrys et al., 2014 |
| Berlin | 3% |  |  |  | 14-16% |  |  |  | Comparison of BS concentrations within and outside LEZ. Adjusted for the changes in traffic intensity. 2008 (with LEZ) compared to 2007 | Lutz, 2009 |
|  |  |  |  |  | 42% (traffic contribution) |  |  | 7-10% | Comparison between 2007 (no LEZ) and 2012 | Lutz, 2013 |
| Bremen | 6% |  |  |  |  |  |  | 6% | No details provided | Reported in Sadler, 2011 |
| Cologne | 7% |  |  |  |  |  |  | 1.5% | Early estimate from monitoring data. PM10 affected by construction works | Reported in Sadler, 2011 |
| Hanover | 1-2% |  |  |  |  |  |  | 5% | No details provided | Reported in Sadler, 2011 |
| Leipzig | No effect (6-15% in summer) |  |  |  | 6-14% (14-29% in summer) |  |  |  | Comparison of annual/summer average concentrations, adjusted wrt reference station | Löschau et al., 2013, cited in Cyrys et al., 2014 |
| Ruhr Area | 4% |  |  |  |  |  |  | 1.2% | Comparison of average concentrations in and out of LEZ | Reported in Sadler, 2011 |
| Munich | 5-12% |  |  |  |  |  |  |  | Ban in through HDV traffic introduced 8 months before LEZ. Analysis based on 4 months monitoring data, adjusted wrt reference station | Cyrys et al., 2009, cited in Cyrys et al., 2014 |
|  | No effect |  |  |  |  |  |  |  | Comparison before and after LEZ, adjustment using reference station data | Morfeld et al., 2013, cited in Cyrys et al., 2014 |
|  | 13% (19.6% in summer; 6.8% in winter) |  |  |  |  |  |  |  | Data for traffic site; 4.5% reduction in annual mean at urban background. Analysis took account of multiple factors using semi-parametric regression model. HDV ban as well as LEZ | Fensterer et al., 2014 |
|  |  |  |  |  | 55% (traffic contribution) |  |  |  | Positive matrix factorization of PM2.5 samples collected before and after LEZ. | Qadir et al., 2013 |
| Milan | No effect | No effect | No effect |  |  |  |  |  | Very short term data. Ratio of BC to PM10 lower in LEZ than outside. | Invernizzi et al., 2011 |
| Amsterdam, The Hague, Den Bosch, Tilburg, Utrecht | No effect | No effect |  |  |  | No effect | No effect | No effect | Comparison before and after LEZ (and in some cases other traffic measures), four suburban stations used as reference stations. | Boogaard et al., 2012 |
| Amsterdam | No effect |  |  |  | 12.9% (limited data) | 7.7% (limited data) | No effect | No effect | Linear regression. Traffic contribution estimated by subtracting data from urban background monitoring site in LEZ. | Panteliadis et al., 2014 |
| Copenhagen |  | 5% |  |  |  |  | No effect |  | Comparison of data from traffic site before and after LEZ. | Jensen et al., 2011 |
| London | No effect | 5-11% (per year) (traffic contribution) |  | 15-17% (per year) (Traffic contribution) |  |  | 3-7% (Traffic contribution) | No effect | Detailed filtering of data to remove confounding factors. Data from sites most likely to be affected by LEZ | Barrett, 2014 |
|  | 1-2% |  |  |  |  |  | No effect |  | Simple comparison of data from sites in and outside LEZ. | Ellison et al., 2013 |

Notes: Sadler (2011) provides a review of the efficacy of LEZs, and is more optimistic than Cyrys et al (2014). Little detail of the methodology used to identify the LEZ effect is given. In this Table data from Sadler (2011) has only been included for those LEZs that there is no other source is readily available.  Data is derived from measurements not modelling.

# Additional References

Bruckmann, P., and M. Lutz. 2010. Verbessern Umweltzonen die Luftqualität? In Tagungsband zum 12. Technischen Kongress des Verbandes der
Automobilindustrie (VDA), 24–25 March 2010, 299–311. Ludwigsburg, Germany: Henrich Druck + Medien GmbH.

Löschau, G., Wiedensohler A., Birmili, W., Rasch, F., Spindler G., Müller K., , Wolf, U., Hausmann, A., Böttger, M.,  Anhalt A., Herrmann, H. 2013. Umweltzone Leipzig, Teil 2: Immissionssituation 2011. Landesamt für Umwelt, Landwirtschaft und Geologie, Dresden, Germany

Morfeld, P., Stern R., Builtjes P., Groneberg D.A., and Spallek M. 2013. Einrichtung einer Umweltzone und ihre Wirksamkeit auf die PM10 Feinstaubkonzentration—eine Pilotanalyse am Beispiel München. Zentralbl. Arbeitsmed. 63:104–15.

Niedermaier, M. 2009. Wirksamkeit von Umweltzonen. ADAC-Untersuchung. ADAC e.V., Interessenvertretung Verkehr. http://www.adac.de/_mmm/pdf/
umweltzonen_wirksamkeit_bericht_0609_43574.pdf (accessed August 8, 2013).

Sadler Consultants Ltd, 2011.  Low Emission Zone in Europe, Report for ADEME, Emmendingen, Germany.